# System Performance Following Loss of Two or more BES Elements

# TPL-003-AB-0

# Standard Effective Date: August 30, 2016

## Audit Summary

| Entity | Entity name; or the ISO. |
| --- | --- |
| Functional Entity Type(s) | Functional entity types applicable to the Entity during the audit period; or the ISO. |
| Audit ID |  |
| Audit Period (Audit start date or standard effective date, whichever comes later) (Audit end date or standard withdrawal/supersede date, whichever comes first) | From: Click or tap to enter a date. To: Click or tap to enter a date. |
| Audit Assessment Start Date (Scheduled or Spot Check) | Click or tap to enter a date. |
| Compliance Monitoring Entity | Alberta Electric System Operator (AESO) for Alberta Entities or WECC on behalf of the Market Surveillance Administrator (MSA) for the ISO. |
| Date of Audit Completion | Click or tap to enter a date. |

The Reliability Standard Audit Worksheet (RSAW) is not an authoritative document. It adds transparency and encourages consistency in the Alberta Reliability Standard monitoring processes. RSAWs provide information related to the compliance assessment approach for each Alberta Reliability Standard. The Compliance Monitoring Entity may apply additional monitoring and assessment methods not documented in the RSAW and reserves the right to request additional evidence not listed in the RSAW. The Entity completes the applicable fields and submits the RSAW to the Compliance Monitoring Entity as requested during a compliance monitoring activity. The Compliance Monitoring Entity completes the document and its compliance assessment in the auditor notes section and retains it as an audit working paper to support the audit conclusions.

### Applicability of Requirements

|  | Req. Removed | ISO | LO AGF | OP AGF | LO GU | OP GU | LO TF | OP TF | LO DS | OP DS | MP DTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 |  | X |  |  |  |  |  |  |  |  |  |
| R2 |  | X |  |  |  |  |  |  |  |  |  |
| R3 |  | X |  |  |  |  |  |  |  |  |  |

LO AGF - Legal owner of an aggregated generating facility

OP AGF - Operator of an aggregated generating facility

LO GU - Legal owner of a generating unit

OP GU - Operator of a generating unit

LO TF - Legal owner of a transmission facility

OP TF - Operator of a transmission facility

LO DS - Legal owner of an electric distribution system

OP DS - Operator of an electric distribution system

MP DTS - Market participant receiving service under Rate DTS of the ISO tariff

### Applicability of Requirements Qualifiers, if applicable:

### Findings

(This section to be completed by the Compliance Monitoring Entity)

| Req. | Finding | Summary and Documentation | Functions Monitored |
| --- | --- | --- | --- |
| R1 |  |  |  |
| R2 |  |  |  |
| R3 |  |  |  |

| Req. | Areas of Concern |
| --- | --- |

| Req. | Recommendations |
| --- | --- |

| Req. | (Positive) Observations |
| --- | --- |

### Subject Matter Experts

Identify the Subject Matter Expert(s) responsible for this Reliability Standard.

Entity Response (Required; Insert additional rows if needed):

| SME Name | Title | Organization | Requirement(s) |
| --- | --- | --- | --- |

## R1 Supporting Evidence and Documentation

R1.	The ISO must demonstrate for transmission facilities rated 100 kV and above, through a planning assessment that the AIES is planned such that the AIES can be operated to accommodate forecasted customer demands, supply and forecasted firm (non- recallable reserved) transmission services, at all demand levels over the range of forecast system demands, under the contingency conditions as defined in Category C of Appendix 1. The controlled interruption of demand to demand customers, the planned removal of generating units, or the curtailment of firm (non-recallable reserved) power transfers may be necessary to meet this reliability standard.

The ISO planning assessments must:

R1.1 Be carried out annually.

R1.2 Be conducted for near term (year one through five) and longer term (year six through ten) planning horizons.

R1.3 Be supported by a study and/or system simulation testing, conducted within the last five years that addresses each of the requirements in requirement R1.3.1 to R1.3.12, showing system performance for the conditions defined in Category C of Appendix 1.

R1.3.1 Be performed and evaluated only for those Category C contingencies that the ISO determines would produce the more severe system results or impacts. The rationale for the contingencies selected for evaluation and an explanation of why the remaining simulations would produce less severe system results must be included in the study.

R1.3.2 Cover critical system conditions and study years as determined necessary by the ISO.

R1.3.3 Be conducted annually unless the ISO determines that changes to system conditions do not warrant such analyses.

R1.3.4 Be conducted beyond the five year planning horizon only as needed to address identified marginal conditions that may have longer lead- time solutions.

R1.3.5 Have all projected firm transfers modeled, if any.

R1.3.6 Be performed and evaluated for selected demand levels over the range of forecast system demands as considered necessary by the ISO.

R1.3.7 Demonstrate that system performance meets the conditions defined in Category C of Appendix 1.

R1.3.8 Include existing and planned facilities as considered necessary by the ISO.

R1.3.9 Include reactive power resources to ensure that adequate reactive resources are available to meet system performance.

R1.3.10 Include the effects of existing and planned protection systems, including any backup or redundant systems.

R1.3.11 Include the effects of existing and planned control devices.

R1.3.12 Include the planned outage and maintenance of any bulk electric equipment (including protection systems or their components) at those demand levels for which planned (including maintenance) outages are performed. This requirement applies only to BES facilities greater than 200 kV or other facilities as specified by the ISO.

R1.4 Address any planned upgrades needed to meet the performance requirements for the conditions defined in Category C of Appendix 1.

R1.5 Consider all contingencies applicable to Category C of Appendix 1.

M1.	The planning assessment will be valid and meet requirements in R1 through the following measures:

(a) The scope of the planning assessment identifies where area studies have been conducted in the past year. It also identifies area studies that have been conducted in previous years and are still valid. Where area studies have not been conducted, a plan and schedule to conduct one is included in the planning assessment.

(b) The planning assessment includes time horizons as specified in R1.2.

(c) The planning assessment has been prepared within the last year.

(d) A certification that the planning assessment complies with each of the R1 technical requirements is provided and states that the planning assessment meets all requirements, identifies requirements not met, and states reasons where the requirement was not met.

(e) A summary list of supporting area studies and needs identification documents is provided. The summary list includes the title and date of the study. The area studies and needs identification documents are provided if requested.

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to TPL-003-AB-0, R1

(This section to be completed by the Compliance Monitoring Entity):

|  | (R1) Confirm that the assessment(s) is a written document based on studies that demonstrate the system will perform as required by the standard. Assessment details should consider: - Critical system conditions - Study years - Projected system demands - Study area - Projected firm transfers - Existing and planned facilities - Reactive power resources - Existing and planned protection systems - Existing and planned control devices - Planned outages - Base cases - Contingencies studied |
| --- | --- |
|  | (R1.1) Assessments should be created every 13 months. |
|  | (R1.2) Confirm that the assessment covers the near term (18 months to 5 years) and long term (6 to 10 years). Confirm forecasted load growth and system configuration changes are properly documented. |
|  | (R1.3) A study is defined by a computer-based analysis using the appropriate seasonal base cases. Confirm that the system modeling for the base case shall be current, including future planned changes, for each of the base case years. Transient stability analysis that demonstrates synchronization between generators with load should be used when steady-state studies demonstrate possible problems. Transient stability studies determine the response of a power system to the occurrence of faults, tripping of a transmission line, tripping of a generator of load shedding. |
|  | (R1.3.1) Confirm that studies performed and evaluated only for those Category D contingencies that the ISO has determined would produce the more severe system results or impacts. The rationale for the contingencies selected for evaluation and an explanation of why the remaining simulations would produce less severe system results must be included in the study. |
|  | (R1.3.2) Confirm that critical system conditions, which for the ISO should include heavy winter and light summer conditions. These conditions may be known to be critical based on engineering judgment and knowledge obtained from numerous studies carried out in the past. Although they are not deemed critical, the light winter, light spring, heavy summer, and heavy spring conditions can also be assessed. |
|  | (R1.3.3) Annually is every 13 months. |
|  | (R1.3.5) Confirm that the assessment included or addressed existing and planned facilities, reactive power resources and the effects of existing and planned protection systems, including any backup or redundant systems. Include the effects of existing and planned control devices. |
|  | (R1.3.6) Confirm that the assessment contains steady-state power flow analysis from different demand levels such as heavy and light summer, heavy and light winter and spring cases. |
|  | (R1.3.7) Assessments should provide plans to rectify the bulk electric system when studies results are outside the criteria defined in Category C of Appendix 1 or do not achieve the required system performance. |
|  | (R1.3.8) Confirm that the assessment includes existing and planned facilities, reactive power resources and the effects of existing and planned protection systems, including any backup or redundant systems. Also the effects of existing and planned control devices should be addressed if applicable. |
|  | (R1.3.9) Confirm that the assessment includes existing and planned facilities, reactive power resources and the effects of existing and planned protection systems, including any backup or redundant systems. Also the effects of existing and planned control devices should be addressed if applicable. |
|  | (R1.3.10) Confirm that the assessment (s) include the effects of existing and planned protection systems, including any backup or redundant systems. |
|  | (R1.3.11) Confirm that the assessment (s) include effects of existing and planned control devices. |
|  | (R1.3.12) Confirm that the assessment(s) include the planned outage and maintenance of any bulk electric equipment (including protection systems or their components) at those demand levels for which planned (including maintenance) outages are performed. |
|  | (R1.4) Confirm that the assessment when applicable addresses plans that include a schedule for implementation, a discussion of expected in-service dates of facilities and lead times necessary to implement plans. |
|  | (R1.5) Confirm that ISO considered or addressed all contingencies applicable to Category C of Appendix 1. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Supporting Evidence and Documentation

R2.	When system simulations indicate an inability of the systems to respond as set out in R1.3.7 of this reliability standard, the ISO must:

R2.1 Provide corrective plans to achieve the required system performance as described above throughout the planning horizon:

R2.1.1 Including a schedule for implementation.

R2.1.2 Including a discussion of expected required in-service dates of facilities.

R2.1.3 Consider lead times necessary to implement plans.

R2.2 Review in subsequent annual assessments where sufficient lead time exists, the continuing need for identified system facilities. Detailed implementation plans are not needed.

M2.	The area studies and needs identification documents contain recommendations and projects that correct the situations where an inability of the systems to respond to requirements specified in R1 has been identified. The area studies and needs identification documents are provided on request.

The area studies and needs identification documents contain the technical components as specified in R2 and its subsections.

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to TPL-003-AB-0, R2

(This section to be completed by the Compliance Monitoring Entity):

|  | (R2) Confirm that plans include a schedule for implementation, a discussion of expected in-service dates of facilities and lead times necessary to implement plans. Are corrective plans involved in any studies and do they resolve the problem? |
| --- | --- |
|  | (R2.1) Corrective plans can be operating procedures or new construction. |
|  | (R2.1.3) Are lead times reasonable for corrective plans? |
|  | (R2.2) When reviewing several assessments for several years (sometimes as many as three), do the studies continue to demonstrate the need for identified system facilities? |
| Note to Auditor: |  |

### Auditor Notes:

## R3 Supporting Evidence and Documentation

R3.	The ISO must provide the planning assessment to WECC on an annual basis.

M3.	A written or email confirmation from WECC that it has received the planning assessment from the ISO. The confirmation includes the date of when the planning assessment was received and source identification information.

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to TPL-003-AB-0, R3

(This section to be completed by the Compliance Monitoring Entity):

|  | (R3) Verify if documentation has been submitted to WECC per WECC’s submission requirements. |
| --- | --- |
| Note to Auditor: |  |

### Auditor Notes:

# Additional Information

Bolded terms in the Alberta Reliability Standard RSAW refer to terms in the Consolidated Authoritative Document Glossary.

# Revision History for RSAW

| Version | Issue Date | Description |
| --- | --- | --- |
| 1.0 | 2026-04-01 | Initial version |